# Contents of the Baseline Emission Inventory (BEI)

# Introduction

As part of the SECAP study, municipalities and local governments aim to develop monitoring and action plans to reduce greenhouse gas (GHG) emissions and decrease final energy consumption. Commitments under the agreement cover the entire geographical area of the local government (town, city, region). In the BEI section dedicated to reducing city emissions, the creation of a Baseline Emission Inventory (BEI) and the identification of emission reduction strategies compared to the current scenario are crucial.

The BEI represents the first stage of the SECAP process and involves calculating the total emissions under the responsibility of the local government for a selected reference year. This study is essential for municipalities to analyze the current situation and determine reduction actions. This document outlines the necessary information for creating the Baseline Emission Inventory and explains the types of activities and consumption to be included in the inventory.

# Baseline Emission Inventory Items

The BEI includes the final energy consumption based on energy sources and sectors during the monitoring year and the corresponding quantities of GHG emissions. This process covers the results of emissions accounting based on current Greenhouse Gas (GHG) methodologies. Local governments should determine the scope of GHG emissions to be included in the inventory by considering local conditions and their jurisdiction. This scope may encompass the geographical boundaries of the city or district the local government’s area of authority. GHG emissions must be reported from at least three main sectors:

- Direct emissions from final energy consumption: Emissions resulting from the use of fuels such as coal, natural gas, and diesel within the local area boundaries. These emissions stem from energy consumption in facilities/buildings (stationary combustion) or transportation vehicles (mobile combustion).

- Indirect emissions related to grid-based energy consumption: Emissions resulting from the consumption of energy sources such as electricity and heat by sectors like residential, commercial, and public buildings within the local area boundaries.

- Non-energy-related emissions: Emissions originating from activities unrelated to energy consumption, such as those from waste management or the livestock/agriculture sectors within the local area boundaries.

A table in the document provides a detailed breakdown of mandatory and optional items for the BEI study, includes explanations, and lists stakeholders and sources from which the necessary data can be obtained.

| 1. Stationary Combustion Inventory |  | Scope |  |  |
| --- | --- | --- | --- | --- |
| 1.1 Stationary Combustion Inventory of Municipal Administration |  |  | Potential Stakeholders and Data Providers | Data Sources. |
| Fuel Consumption in Municipal Facilities | Fuel consumption (e.g., natural gas, coal, diesel) for combustion processes in facilities owned by the municipal administration. This data must be collected from all municipal facilities through invoices or inventory studies. | Mandatory | Municipal energy management unit, accounting inventory, etc. | Internal inventory |
| Electricity Consumption in Municipal Facilities | Electricity consumption in facilities owned by the municipal administration. This data must be collected from all municipal facilities through electricity bills or meters. | Mandatory | Municipal energy management unit, accounting inventory, etc. | Internal inventory or can be obtained from the relevant distribution company using meter ID. |
| Lighting System Usage and Electricity Consumption in Municipal Facilities | Inventory of lighting systems in municipal facilities. While optional, it is useful for energy audits. | Mandatory | Municipal energy management unit, accounting inventory, etc. | Internal inventory or can be obtained from the relevant distribution company using meter ID. |
| Generator Usage in Municipal Facilities | Fuel consumption and electricity generation data of cogeneration units and generators in municipal facilities. | Mandatory | Municipal energy management unit, accounting inventory, etc. | Internal inventory |
| Energy Production Plants Owned by Municipal Administration | Electricity generation data from solar plants, biogas plants, or waste-to-energy plants owned by the municipality. Plant characteristics should also be shared. | Mandatory | Municipal energy management unit, accounting inventory, etc. | Internal inventory |
| 1.2 Stationary Combustion Inventory at Urban Scale |  | Scope | Potential Stakeholders and Data Providers | Data Sources. |
| Fuel Consumption in Residential Housing Stock | Fuel consumption for heating (e.g., natural gas, LPG, coal) in the residential housing stock. | Mandatory | Data on external inventory can be accessed via EPDK and TÜİK sources. | EPDK Doğalgaz Piyasa Raporu EPDK Petrol ve LPG Piyasa Raporu |
| Electricity Consumption in Residential Housing Stock | Electricity consumption in residential/household settings. | Mandatory | Data on external inventory can be accessed via EPDK and TÜİK sources. | EPDK Elektrik Sektör Raporu |
| Fuel Consumption in Commercial and Public Buildings | Fuel consumption (e.g., natural gas, coal) in commercial and public buildings. | Mandatory | Data on external inventory can be accessed via EPDK and TÜİK sources. | EPDK Doğalgaz Piyasa Raporu EPDK Petrol ve LPG Piyasa Raporu |
| Electricity Consumption in Commercial and Public Buildings | Electricity consumption in commercial and public buildings. | Mandatory | Data on external inventory can be accessed via EPDK and TÜİK sources. | EPDK Elektrik Sektör Raporu |
| Electricity Consumption in Lighting | Electricity consumption for urban lighting. | Mandatory | Regional electricity distribution companies are responsible for streetlights and lighting infrastructure and are the data owners. | EPDK Elektrik Sektör Raporu |
| Fuel Consumption in Industrial and Manufacturing Sectors | If no action will be taken for this sector within the scope of the SECAP study, it is recommended to exclude it. | Optional | Data on external inventory can be accessed via EPDK and TÜİK sources. | EPDK Doğalgaz Piyasa Raporu EPDK Petrol ve LPG Piyasa Raporu |
| 2. Hareketli Yakma Envanteri |  |  |  |  |
| 1.1. Belediye Yönetimine Ait Hareketli Yakma Envanteri |  | Scope | Potential Stakeholders and Data Providers | Data Sources |
| Fuel Consumption of Municipal Vehicle Fleet | Inventory of all vehicles owned or leased by the municipality, including those allocated to employees or used for operational services (e.g., garbage collection). The fleet should be categorized by vehicle type (passenger, heavy-duty, etc.) and fuel type (gasoline, diesel, electric, etc.). | Mandatory | All municipal affiliates | Internal inventory |
| Fuel Consumption of Municipal Public Transportation Vehicles | Fuel types and quantities for buses and minibuses operated by the municipality. | Mandatory | Public transportation department | Internal inventory |
| Fuel Consumption of Municipal Rail Systems | Electricity and other fuel consumption of tram, metro, and similar rail systems operated by the municipality. | Mandatory | Rail Systems Directorate, Public Transportation Department | Internal inventory |
| Fuel Consumption of Municipal Maritime Transportation | Fuel consumption of ferries, boats, or other maritime vehicles used for public transportation or marine cleaning operated by the municipality. | Mandatory | Maritime Lines Directorate, Public Transportation Department | Internal inventory |
| Off-road Vehicle Fuel Consumption | If no action will be taken for this sector within the scope of the SECAP study, it is recommended to exclude it. | Optional |  | Internal inventory |
| 1.2. Mobile Combustion Inventory at Urban Scale |  | Scope | Potential Stakeholders and Data Providers | Data Sources |
| Fuel Consumption of Private and Commercial Vehicles in the City | Total fuel consumption of vehicles used by residents for personal or commercial purposes. | Mandatory | Data on external inventory can be accessed via EPDK and TÜİK sources, using the total vehicle stock and fuel sales data in the city. | EPDK Doğalgaz Piyasa Raporu EPDK Petrol ve LPG Piyasa Raporu |
| Fuel Consumption in Public Transportation Vehicles Operated by Private Entities | Total fuel consumption of buses, minibuses, and taxis operated by private companies for public transport services. | Mandatory | Private transportation operators, transportation cooperatives. | Data on external inventory can be accessed via EPDK and TÜİK sources, including operator reports. |
| Fuel Consumption in Urban Rail Systems Operated by Private Entities | Electricity and/or fuel consumption of metro, tram, or light rail systems operated by private companies. | Mandatory | Private operators (if applicable) | Internal operator records or through municipal reporting systems. |
| Fuel Consumption in Maritime Transportation Operated by Private Entities | Total fuel consumption of ferries, boats, or other water-based transportation systems operated by private companies. | Optional | Private operators (if applicable) | EPDK Doğalgaz Piyasa Raporu EPDK Petrol ve LPG Piyasa Raporu |
| Off-road Vehicle Fuel Consumption for Urban Activities | Total fuel consumption of construction vehicles, machinery, and other off-road vehicles used in urban activities. | Optional | Construction firms, industrial zones, or rental companies | İlgili iştirake ait envanter |
| 3. Emissions from Agricultural, Forestry, Fishery, and Livestock Activities |  | Scope | Potential Stakeholders and Data Providers | Data Sources |
| Emissions from Agricultural and Forestry Activities | If no action is to be taken in the SECAP study for the sector, it is recommended to exclude this scope. | Optional | Tarım Orman Bakanlığı ve TUIK | TUIK |
| Emissions from Fishery Activities | If no action is to be taken in the SECAP study for the sector, it is recommended to exclude this scope. | Optional | Tarım Orman Bakanlığı ve TUIK | TUIK |
| Emissions from Livestock Activities | If no action is to be taken in the SECAP study for the sector, it is recommended to exclude this scope. | Optional | Tarım Orman Bakanlığı ve TUIK | TUIK |
| 4. Emissions from Waste Management |  | Scope | Potential Stakeholders and Data Providers | Data Sources |
| Solid Waste Disposal | Details of how much of the city’s solid waste is disposed of should be listed. | Mandatory | Waste Management Branch Directorate, Environmental Engineering Units | Internal inventory |
| Biological Waste Treatment | Information about biological waste facilities and waste treatment amounts should be shared. | Mandatory | Waste Management Branch Directorate, Environmental Engineering Units | Internal inventory |
| Waste Incineration and Open Burning | Detailed information about open or closed burning processes and the total amount of waste burned should be shared. | Mandatory | Waste Management Branch Directorate, Environmental Engineering Units | Internal inventory |
| Wastewater Treatment and Discharge | Information about wastewater treatment facilities and their total treatment capacity should be shared. | Mandatory | Waste Management Branch Directorate, Environmental Engineering Units | Internal inventory |
| Recycling | Details about how much of the city’s waste (e.g., metal, glass, plastic) is being recycled should be listed. | Mandatory | Waste Management Branch Directorate, Environmental Engineering Units | Internal inventory |