# PRACOWNIA 44STO SP. Z OO

# based in Gliwice

# st. Ziemowita 17/4, 44 100 Gliwice

# NIP: 631-266-70-42

MODEL SOLUTIONS FOR DEVELOPMENT OF BUS STOPS IN KATOWICE 
Design concepts using nature-based solutions (NBS) 
as part of the UPSURGE project

The design team of Pracownia 44STO:

MSc. Eng. Marta Gocek, landscape architect

MSc. Eng. Ewa Twardoch, landscape architect

MSc. Eng. arch. Katarzyna Herman

Dr. Eng. arch. Agata Twardoch

Purchaser:

City of Katowice

Municipal Greenery Authority in Katowice

GENERAL ASSUMPTIONS :

- Increasing micro-retention and reducing surface runoff by removing concrete from the surface as much as possible, while maintaining their functionality, and planting plants that inhibit or delay the outflow of rainwater

- Reducing the urban heat island by planting trees, building trellises planted with vines, removing concrete from the surface, planting sodding plants

- Increasing biodiversity by planting diverse plants that are resistant and do not require care, but are attractive to fauna, melliferous plants, etc.

- Improving air quality by introducing plants that absorb dust

- Improving the comfort of space users by:

- introducing shields from the sun, wind and rain,

- setting small architecture adapted to various needs

- users to increase aesthetic values

# Ochojec Hospital stop

## ain conceptual assumptions

- Designing two bus shelters with roofs covered with greenery

- Designing diverse street furniture: benches, squats, litter bins

- Design of ramps and renovation of stairs

- Planting the escarpment and creating a sensory rebate at the top of the escarpment

- Planting several trees

## Expected benefits

- The introduction of diverse street furniture and ramps will increase the comfort of users with special needs

- Planting the escarpment with diversified vegetation will increase the visual attractiveness of the area and biodiversity

- Planting a tree over the years will increase the shaded area, which will reduce the heat island effect

# stop / Silesian Parliament Square

## Main conceptual assumptions

- Adaptation to the "Concept for the development of Plac Śląski in Katowice" by Meritum Projekt

- Enlargement of the existing rebates and unconcreting the maximum area of pavements, without negatively affecting their functionality

- Unconcreting, if possible, the surface under the candelabra balusters

- Planting attractive bedding plants, resistant to urban conditions, insect-friendly and melliferous

- Tree planting

## Expected benefits

- Increasing the areas covered with vegetation will reduce surface runoff of rainwater and increase small retention

- Diversified plantings will increase the biodiversity and visual attractiveness of the site

- Planting a tree over the years will increase the shaded area, which will reduce the heat island effect

# Market Square - ul. Korfanty

## Main conceptual assumptions

- Creation of new rebates referring in shape to the existing development of the Market Square in Katowice and inscribing them in the surface drawing

- Maximum unconcreting of the surface, while maintaining its functionality, will reduce surface runoff and increase low retention

- Planting trees to minimize the urban heat island in the future

- Planting resistant grasses, perennials and shrubs will increase the visual value and increase biodiversity

## Expected benefits

- Increasing the areas covered with vegetation will reduce surface runoff of rainwater and increase small retention

- Diversified plantings will increase the biodiversity and visual attractiveness of the site

- Planting a tree over the years will increase the shaded area, which will reduce the heat island effect

# Stop ul. Chorzowska Oak - Church

## Main conceptual assumptions

- Designing bus shelters with roofs covered with greenery and two trellises covered with vines

- Planting the rebate at the back and side of the stop with resistant plants

- Planting the area between the bicycle path and the tracks

## Expected benefits

- Trellises overgrown with vines will increase shading and thus the comfort of using the space

- Increasing the areas covered with vegetation will reduce surface runoff of rainwater and increase small retention

- Diversified plantings will increase the biodiversity and visual attractiveness of the site

# Silesian Park stop

## Main conceptual assumptions

- Designing two bus shelters with roofs covered with greenery

- Designing diverse street furniture: benches, squats, litter bins

- Designing a trellis covered with vines

- Maximum unconcreting of the surface, while maintaining its functionality, will reduce surface runoff and increase low retention

- Planting resistant grasses, perennials and shrubs will increase the visual value and increase biodiversity

- Planting a tree

## Expected benefits

- The introduction of diverse street furniture and ramps will increase the comfort of users with special needs

- Increasing the areas covered with vegetation will reduce surface runoff of rainwater and increase small retention

- Diversified plantings will increase the biodiversity and visual attractiveness of the site

- Planting a tree over the years will increase the shaded area, which will reduce the heat island effect